Name:Dhananjay madhukar bhirud

Roll No: 152

Practical No.: 5 Write a Python script to perform image filtering in frequency domain.

import cv2

importnumpy as np

importmatplotlib.pyplot as plt

defcreate_filter(shape, filter_type='low_pass', cutoff=30):

rows, cols = shape

crow, ccol = rows // 2, cols // 2

filter_mask = np.zeros((rows, cols), np.float32)

iffilter_type == 'low_pass':

cv2.circle(filter_mask, (ccol, crow), cutoff, 1, thickness=-1)

eliffilter_type == 'high_pass':

filter_mask = np.ones((rows, cols), np.float32)

cv2.circle(filter_mask, (ccol, crow), cutoff, 0, thickness=-1)

returnfilter_mask

deffilter_frequency_domain(image, filter_type='low_pass', cutoff=30):

# Convert image to float32 for precision

image = np.float32(image)

# Perform FFT

dft = cv2.dft(image, flags=cv2.DFT_COMPLEX_OUTPUT)

dft_shift = np.fft.fftshift(dft)

# Create filter mask

filter_mask = create_filter(image.shape, filter_type, cutoff)

# Apply filter

filtered_dft_shift = dft_shift * filter_mask[..., np.newaxis]

# Inverse FFT

dft_ishift = np.fft.ifftshift(filtered_dft_shift)

img_back = cv2.idft(dft_ishift)

img_back = cv2.magnitude(img_back[:, :, 0], img_back[:, :, 1])

# Normalize the result

img_back = np.abs(img_back)

img_back = cv2.normalize(img_back, None, 0, 255, cv2.NORM_MINMAX)

img_back = np.uint8(img_back)

returnimg_back

def main():

# Read the image

image = cv2.imread('E:\\Msc\\leaf.jpg', cv2.IMREAD_GRAYSCALE)

if image is None:

print("Error: Image not found.")

return

# Parameters

low_pass_cutoff = 30

high_pass_cutoff = 30

# Apply Low Pass Filter

low_pass_filtered_image = filter_frequency_domain(image, 'low_pass', low_pass_cutoff)

# Apply High Pass Filter

high_pass_filtered_image = filter_frequency_domain(image, 'high_pass', high_pass_cutoff)

# Display the original and filtered images

plt.figure(figsize=(18, 6))

plt.subplot(1, 3, 1)

plt.title('Original Image')

plt.imshow(image, cmap='gray')

plt.subplot(1, 3, 2)

plt.title('Low Pass Filtered Image')

plt.imshow(low_pass_filtered_image, cmap='gray')

plt.subplot(1, 3, 3)

plt.title('High Pass Filtered Image')

plt.imshow(high_pass_filtered_image, cmap='gray')

plt.show()

if __name__ == "__main__":

main()

Output